RAPTOR

Onboard Perception Design and Daily Work Report

16 September 2026, with the 17 September addendum

This report combines the two documents written on 16 September — the design report (2026-09-16-raptor-jetson-perception-design.md) and the daily work report (RAPTOR_Work_Report_2026-09-16_2254.docx) — into one. The wording is kept as written at the time: later reports supersede parts of it (for example, the board runs JetPack 7.2 and ROS 2 Jazzy, not JetPack 6 and Humble, and both model choices have since changed).

| Field | Value |
| --- | --- |
| Date | Wednesday, 16 September 2026 |
| Reports combined | Design report (.md) and daily work report generated 2026-09-16 22:54 (.docx) |
| Author | Majd Awad |
| Project | RAPTOR — search-and-rescue UAV, AlfaisalX |
| Phase | Phase 0 — Foundations (design complete, no pipeline code yet) |
| Repository | github.com/aniskoubaa/alfaisalx_students_projects |

# 1. Summary of the day

This session took RAPTOR from a one-paragraph README to a complete, argued design for the onboard compute stack, resolved the hardware unknowns that were blocking it, and began environment bring-up on the Jetson.

What the system does. RAPTOR is a search-and-rescue quadcopter. For every person it finds it produces a geolocated victim report carrying a snapshot, a posture, a free-text description, and a triage flag for a human operator to review.

The core decision. The three questions the system must answer — is someone there, what posture are they in, what is happening to them — cost wildly different amounts of compute. So they get three tiers, and the expensive one is gated:

| Tier | Question | Method | Rate |
| --- | --- | --- | --- |
| 1 | Is there a person? | YOLO11-pose + ByteTrack, TensorRT INT8 | every frame, ~30 fps |
| 2 | What posture? | Gravity-aligned keypoint classifier (ours) | every frame, <1 ms |
| 3 | What is happening? | Qwen2.5-VL-3B at INT4, event-triggered | ~0.2 Hz |

A 3B VLM on an Orin NX produces tens of tokens per second — seconds per description. It cannot run at frame rate and does not need to. Tiers 1–2 decide when spending those seconds is worth it. This gating is the single decision the whole architecture depends on.

Status. Design complete and documented. Hardware identified and specified. No pipeline code written. Phase 0 (flash, ROS 2, camera and MAVROS publishing) is the next step and is currently blocked only on a 12 V bench power supply for the camera.

# 2. Hardware — confirmed this session

| Component | Confirmed spec | How confirmed |
| --- | --- | --- |
| Companion computer | Jetson Orin NX 16 GB | free -h reported 15Gi |
| Flight controller | Pixhawk 6C | Stated by team |
| Camera | SIYI A8 mini 4K, 3-axis gimbal, 95 g | Stated by team |
| Camera optics | Sony 1/1.7″ 8 MP, f/2.8, 81° H / 93° D FOV | A8 mini User Manual v1.10 |
| Camera video path | RTSP over Ethernet, 192.168.144.25:8554/video1 (superseded 2026-09-17 — see section 10) | Manual v1.10, p.62 |
| Camera power | 11–25.2 V (3S–6S), 5 W avg / 12 W peak | Manual v1.10, p.16 |
| Camera USB-C | Firmware upgrade only — not power, not video | Manual + empty dmesg on the Jetson |
| Gimbal control | SIYI SDK over UDP, same Ethernet link | Manual v1.10 |
| Recording | 4K @ 25 fps H.265 to microSD (zoom disabled in 4K) | Manual v1.10, p.17 |

The 16 GB board settles the model choice: Qwen2.5-VL-3B at INT4 fits comfortably alongside the TensorRT engines and ROS 2. The 8 GB fallback ladder is not needed.

## Harness

Every device taps the battery in parallel. Nothing routes through the Pixhawk.

LiPo main lead (XT60) ─┬─> power module ──(6-pin JST-GH)──> Pixhawk POWER1   [5 V + sensing]

├─> ESCs / motors                                     [pack voltage]

├─> SIYI A8 mini power cable                          [11–25.2 V, own branch]

└─> DC regulator ─────────────────> Jetson Orin NX    [carrier-dependent]

LiPo balance lead (5-pin JST-XH) ──> charger only. Never a power source.

Data:  A8 mini ──Ethernet──> Jetson        [RTSP video + gimbal UDP, one cable]

Pixhawk TELEM1 ──────> Jetson        [MAVLink / MAVROS, read-only]

A8 mini UART ────────> TELEM2        [optional: attitude feed + photo EXIF geotag]

Interactive version: RAPTOR/docs/raptor-wiring-map.html

## Four mistakes identified and avoided

- The 5-pin balance lead is not a power source. It is a per-cell tap for the charger. Thin wire, and which pins you use decides whether you get 3.7, 7.4, 11.1 or 14.8 V.

- The Pixhawk POWER port is an input. It takes 5 V into the flight controller from the power module. There is no 12 V rail to draw from, and it cannot feed a 12 W gimbal.

- USB-C will not power or stream the camera. Firmware upgrades only. Confirmed by an empty dmesg on the Jetson.

- Do not run the camera through the PDB. SIYI is explicit: power the gimbal directly from a 3S–6S battery if the aircraft rolls hard.

# 3. Design decisions and rationale

## 3.1 Detector — YOLO11-pose

One pose model rather than a detector plus a pose model: the -pose variant already emits person boxes, so running both would double the cost for nothing. ByteTrack for identity because it keeps low-confidence boxes alive, which is exactly the regime small aerial targets live in. TensorRT INT8, calibrated on our own footage rather than COCO.

Rejected: YOLOv8 (superseded), RT-DETR (heavier at our input sizes, fiddlier to export — but the fallback if licensing forces it), NVIDIA PeopleNet (ground-level viewpoint bias), any cloud API (violates the offline constraint).

Open licensing risk: Ultralytics YOLO is AGPL-3.0. Acceptable for an open academic project, contagious if RAPTOR ships to a third party without source. This must be decided before code is built on it.

## 3.2 Posture — a small model we build ourselves

Posture is a geometry question and gets a geometric answer: the torso vector from 17 COCO keypoints. Deterministic, explainable to an operator, effectively free, and it cannot hallucinate. Asking a VLM the same question costs seconds and sometimes invents an answer.

The non-obvious part is gravity alignment. During a banked turn the whole image rotates, so a standing person's torso reads as horizontal and a naive classifier raises a false casualty. We rotate keypoints into a gravity-aligned frame using the Pixhawk's own roll estimate, arriving over MAVROS, before classifying.

No public model does this, because no public model has our IMU stream. A two-layer MLP on normalised keypoints, under 100 KB, trainable on a few thousand labelled frames. This is the genuinely novel and publishable component of the project.

## 3.3 Scene understanding — a VLM, not an LLM

A text-only LLM cannot see the camera; the best it could do is paraphrase YOLO's labels, which adds nothing. The value — dark staining on the trouser leg, an arm bent unnaturally, trapped under debris — lives in pixels no detector describes.

| Model | Role | Reasoning |
| --- | --- | --- |
| Qwen2.5-VL-3B | Primary, INT4 | Fits in ~2–2.5 GB. Best small model for fine visual detail, real region grounding so it can be pointed at our detected person, native dynamic resolution so small crops are not squashed. Action required: verify the specific checkpoint's licence — the family is not uniformly Apache. |
| VILA-1.5-3B | Fallback 1 | NVIDIA-optimised for Jetson; easiest deployment path. |
| SmolVLM2 | Fallback 2 | Apache-2.0, tiny and fast; shallower descriptions. |
| Moondream2 | Fallback | Small and permissively licensed. |
| GPT-4o / Claude / Gemini | Rejected | Best quality, but a disaster area has no reliable link. Optional ground-station enrichment only, never flight-critical. |
| LLaVA-1.6 7B | Rejected | Poor size-to-quality trade against a 2025-era 3B. |
| Florence-2 | Rejected as primary | Not instruction-following. |

Two rules keep it honest: ground it — send the person crop with a 30 % context margin plus the structured facts tiers 1–2 already established; and constrain it — grammar-forced JSON against a fixed schema, including a not_visible field so uncertainty has somewhere to go other than invention.

## 3.4 Build, fine-tune, or use off the shelf

| Component | Verdict |
| --- | --- |
| Person detector | Fine-tune. COCO-pretrained backbone on aerial SAR data. Training from scratch would be strictly worse and cost a semester. |
| Pose estimator | Use off the shelf. |
| Posture classifier | Build it. Needs our IMU-derived gravity alignment; nothing public has that. |
| Action recognition | Defer. Posture + motion may already cover the critical cases. |
| VLM | Use off the shelf. We lack the data to fine-tune, and a LoRA on a few hundred staged casualties would learn our volunteers' clothing, not injury. |

Fine-tuning the detector is not optional: stock YOLO trained on ground-level photographs systematically misses prone people seen from above, which is the exact failure we can least afford. Candidate datasets: HERIDAL, SARD, Okutama-Action, VisDrone, TinyPerson.

## 3.5 ROS 2

Five nodes on Humble: camera_node, detector_node, vlm_node, triage_aggregator_node, mavros. The detector and VLM are separate nodes on separate CUDA streams, so a four-second inference cannot stall perception — and if the VLM node dies, the system degrades to posture-only reporting rather than going blind.

Recording every topic to rosbag2 on every flight is what makes the rest tractable: benchmarks replay identical bags instead of demanding new flights, and the same bags become the labelled dataset, complete with the synchronised GPS and attitude the posture classifier depends on — which cannot be recovered afterwards.

# 4. Key finding: the altitude ceiling is lower than assumed

The A8 mini's 81° horizontal FOV gives a focal length of ~1124 px across a 1920 px frame. Geometric prediction, nadir view, no zoom:

| Altitude | Standing (1.7 m) | Prone from overhead (0.45 m) | Ground swath |
| --- | --- | --- | --- |
| 20 m | 96 px | 25 px | 34 m |
| 25 m | 76 px | 20 px ← detection floor | 43 m |
| 30 m | 64 px | 17 px | 51 m |
| 40 m | 48 px | 13 px | 68 m |
| 60 m | 32 px | 8 px | 103 m |

Against a practical detection floor of ~20 px, a prone casualty falls below it at roughly 25 m. The wide lens buys coverage and costs pixels.

If this holds in measurement it is the most operationally important number the project will produce, and it forces a choice: fly lower than the coverage-optimal altitude, or adopt tiled inference (experiment E4). It must be confirmed against the streamed resolution, which may be below 1920 px.

Note: earlier drafts of the design docs assumed a 60° FOV. Those figures were corrected on 2026-09-16 once the manual was consulted. The real optics make the problem harder, not easier.

# 5. Environment bring-up — started

Work begun on the Jetson the same day, ahead of the camera arriving on power.

Resolved: `externally-managed-environment` (PEP 668). JetPack 6 is Ubuntu 22.04, which blocks pip install into the system Python. Fixed with a virtualenv:

python3 -m venv --system-site-packages ~/raptor-venv

source ~/raptor-venv/bin/activate

--system-site-packages is mandatory, not stylistic: JetPack's CUDA-enabled PyTorch, TensorRT and OpenCV live in the system site-packages and cannot be reinstalled from PyPI on aarch64, where the torch wheel is CPU-only. A sealed venv imports cleanly and silently never touches the GPU. --break-system-packages was rejected as a way to corrupt the install.

Standing check after any install that pulls torch (Ultralytics does):

python -c "import torch; print(torch.cuda.is_available())"

Must print True. Ultralytics lists torch as a dependency and pip can quietly substitute a CPU wheel.

Reboot behaviour recorded.

| Lost on reboot | Restore with |
| --- | --- |
| Venv activation | source ~/raptor-venv/bin/activate |
| jetson_clocks | sudo jetson_clocks — never persists, by design |
| Camera static IP | sudo ip addr add 192.168.144.30/24 dev eth0, or netplan |

nvpmodel does persist. Model weights are ordinary files and persist. Note that yolo is a one-shot command, not a background service — which is why the production system will be a long-running ROS 2 node. Full table and the back-to-work sequence in docs/09-jetson-environment.md.

Next measurement: yolo benchmark model=yolo11n-pose.pt imgsz=640 at MAXN with tegrastats sampling, giving the FP16 PyTorch baseline for experiment E1. Not yet run.

# 6. Current blocker

pip install ultralytics reports numpy and scipy not installed. Diagnose first — the venv may be sealed off from JetPack's packages:

grep system-site ~/raptor-venv/pyvenv.cfg

# must read: include-system-site-packages = true

source ~/raptor-venv/bin/activate

pip install "numpy<2" scipy

python -c "import torch, numpy; print(torch.__version__, numpy.__version__, torch.cuda.is_available())"

Pin numpy below 2.0. NVIDIA's Jetson PyTorch wheels are built against numpy 1.x, and numpy 2.0 changed the C ABI — installing numpy 2 is a common way to silently break CUDA torch on this board. (Later superseded: the torch that was eventually installed is built for NumPy 2 — see the 28 September report.)

Second blocker: the camera has never been powered. Waiting on a 12 V bench supply (2 A current limit). No ping, no stream and no decode has been tested.

# 7. Deliverables produced

| File | Contents |
| --- | --- |
| ProgressReports/2026-09-16-raptor-jetson-perception-design.md | Full design report (combined into this document) |
| ProgressReports/STATUS.md | Living where-we-stopped checkpoint |
| RAPTOR/docs/README.md | Index and short version |
| RAPTOR/docs/01-system-overview.md | Mission, hardware, constraints, data flow |
| RAPTOR/docs/02-detection-and-pose.md | Detector and posture choices, rejected alternatives |
| RAPTOR/docs/03-scene-understanding-vlm.md | VLM selection, prompting, output schema |
| RAPTOR/docs/04-ros2-architecture.md | Node graph, topics, custom messages |
| RAPTOR/docs/05-custom-models-and-data.md | Build vs. fine-tune vs. use; datasets; evaluation |
| RAPTOR/docs/06-benchmark-plan.md | Experiments E1–E8, performance budget, method |
| RAPTOR/docs/07-roadmap.md | Phases 0–6, milestones, risks |
| RAPTOR/docs/08-limitations-and-safety.md | Failure modes, privacy, GACA, model card |
| RAPTOR/docs/09-jetson-environment.md | Venv, PEP 668, CUDA torch, board commands |
| RAPTOR/docs/raptor-explainer.html | Animated explainer — live pipeline sim, interactive figures |
| RAPTOR/docs/raptor-wiring-map.html | Interactive harness diagram and connector reference |

Six commits pushed to main at github.com/aniskoubaa/alfaisalx_students_projects (a9f346f..f3944c8), approximately 4,000 lines.

# 8. Open questions

| # | Question | Blocks |
| --- | --- | --- |
| 1 | Which carrier board is the Orin NX on? | Regulator output voltage — 5 V, 12 V or 19 V |
| 2 | ArduPilot or PX4 on the Pixhawk 6C? | The optional gimbal UART parameters |
| 3 | Actual streamed resolution and frame rate over RTSP | The altitude table above; detector input sizing |
| 4 | H.264 or H.265 on the stream? | GStreamer pipeline elements |
| 5 | Is the AGPL-3.0 licence on Ultralytics acceptable? | Whether we build on YOLO11 at all |
| 6 | Rename jetson_orin_nano_benchmarks/? | Flight computer is an NX, not a Nano |

# 9. Next steps — Phase 0

Blocked on one purchase: a bench DC supply (12 V, 2 A limit). Current limiting means a wiring mistake trips the supply instead of destroying the camera. Immediately, in order:

1.  Fix the numpy/scipy blocker; confirm torch.cuda.is_available() is True.

2.  Run yolo checks; confirm it names the GPU rather than cpu.

3.  Measure the FP16 PyTorch baseline: yolo benchmark model=yolo11n-pose.pt imgsz=640, at MAXN, with tegrastats sampling in a second terminal.

4.  Power the camera alone. Confirm the three-axis self-test sweep and a solid green LED. (Yellow blinking = input under 10 V.)

5.  sudo ip addr add 192.168.144.30/24 dev eth0, then ping 192.168.144.25.

6.  ffplay rtsp://192.168.144.25:8554/video1 — record the true resolution and frame rate.

7.  Prove hardware decode: a GStreamer pipeline using nvv4l2decoder.

8.  Flash JetPack 6.x; record CUDA / TensorRT / cuDNN versions.

9.  Install ROS 2 Humble; talker/listener.

10.  Pixhawk TELEM1 → Jetson; bring up MAVROS; confirm GPS and attitude as topics.

11.  Sync the Jetson clock to flight-controller time.

12.  Record a rosbag and replay it. This is the Phase 0 finish line.

Steps 8–11 need no camera and can start immediately.

Start data-collection planning in parallel. It is the long pole for Phases 3–5: the consent form, the flight plan across three altitudes and three lighting conditions, and the labelling workflow all take longer than the code.

# 10. Addendum — 17 September: video path changed

The camera-to-Jetson link was changed from Ethernet/RTSP to micro-HDMI through a USB capture card, freeing the camera's Ethernet port to feed a video transmitter for the operator downlink.

A8 mini micro-HDMI --> USB capture card --> Jetson      [perception video]

A8 mini Ethernet   --> video transmitter --> operator   [downlink]

A8 mini UART       --> Jetson serial                    [gimbal control]

What this buys. The operator sees the camera directly rather than a stream relayed by the Jetson, so a Jetson fault no longer blinds them. Capture is also simpler to bring up: a UVC device at /dev/video0 with no network configuration.

What it costs.

- Gimbal control had to move to UART. It previously shared the Ethernet link. The A8 mini supports the SIYI SDK over UART, so this works, but it is a wiring change.

- Frames now arrive in host memory. The RTSP path could hardware-decode straight into GPU memory via nvv4l2decoder; a capture card forces a host-to-GPU copy every frame.

- The card adds its own latency, typically tens to well over a hundred milliseconds, invisible unless deliberately measured. Experiment E3b was added to the benchmark plan.

- USB 3.0 is required. Uncompressed 1080p30 is about 62 MB/s; a USB 2.0 port silently degrades to MJPEG, 720p or a lower frame rate rather than failing.

- The ground picture carries no detection overlays, since it bypasses the Jetson.

Unresolved risk. The manual describes video output as a switchable mode — “switch video output mode to HDMI under the Gimbal Config page”. If enabling HDMI disables the Ethernet stream, this split harness is not possible. Verify on the bench before buying a capture card or transmitter.

Also confirmed from the manual: any live output from the A8 mini — Ethernet, HDMI or CVBS — is 1080p maximum. The 4K figure applies only to SD-card recording, so 1920 px remains the correct basis for the altitude table in section 4.

# 11. Scope boundary and standing caveat

RAPTOR perceives and reports. It does not fly the aircraft, plan paths, or command the flight controller — the Jetson reads telemetry from MAVROS and never writes control commands. Enforced in code, so a perception bug can never become a flight safety incident.

The system is a search aid, not a medical device and not a triage system. The VLM produces observations; the triage flag is computed by explicit, auditable rules over structured fields, each carrying a plain-words reason. A human decides what happens. Placeholder thresholds need review by someone with real SAR or emergency-medicine experience before any operational use. Full treatment in docs/08-limitations-and-safety.md.

Every performance figure in this report is a target, not a measurement. Nothing had been benchmarked yet. The altitude table is geometry, not data. The benchmark plan exists to replace these figures; until it does, none of them belongs in a presentation as a finding.