RAPTOR — Daily Work Report

Onboard perception design and Jetson environment bring-up

| Field | Value |
| --- | --- |
| Date | Wednesday, 16 September 2026 |
| Report generated | 2026-09-16 22:54 |
| Author | Majd Awad (majd.awad2016@gmail.com) |
| Project | RAPTOR — search-and-rescue UAV, AlfaisalX |
| Phase | Phase 0 — Foundations (design complete, no pipeline code yet) |
| Repository | github.com/aniskoubaa/alfaisalx_students_projects |

# 1. Summary of the day

Today took RAPTOR from a one-paragraph README to a complete and argued design for the onboard compute stack, resolved every hardware unknown that was blocking it, and began environment bring-up on the Jetson. Fifteen documentation files were written and pushed to GitHub.

RAPTOR is a search-and-rescue quadcopter. For every person it finds, it produces a geolocated victim report carrying a snapshot, a posture, a description and a triage flag for a human operator to review. The airframe and flight controller already exist; all work is on the Jetson Orin NX.

## The central design decision

The three questions the system must answer cost wildly different amounts of compute, so they are answered in three tiers — and the expensive tier is gated:

| Tier | Question | Method | Rate |
| --- | --- | --- | --- |
| 1 | Is there a person? | YOLO11-pose + ByteTrack, TensorRT INT8 | every frame, ~30 fps |
| 2 | What posture? | Gravity-aligned keypoint classifier (ours) | every frame, <1 ms |
| 3 | What is happening? | Qwen2.5-VL-3B at INT4, event-triggered | ~0.2 Hz |

A 3B vision-language model on an Orin NX produces tens of tokens per second — seconds per description. It cannot run at frame rate and does not need to. Tiers 1 and 2 run on every frame and decide when spending those seconds is worthwhile. This gating is the single decision the whole architecture depends on.

# 2. Hardware confirmed today

| Component | Confirmed specification | How confirmed |
| --- | --- | --- |
| Companion computer | Jetson Orin NX 16 GB | free -h reported 15Gi |
| Flight controller | Pixhawk 6C | Stated by team |
| Camera | SIYI A8 mini 4K, 3-axis gimbal, 95 g | Stated by team |
| Camera optics | Sony 1/1.7in 8 MP, f/2.8, 81 deg H / 93 deg D FOV | User Manual v1.10 |
| Camera video | RTSP over Ethernet, 192.168.144.25:8554/video1 | Manual v1.10, p.62 |
| Camera power | 11-25.2 V (3S-6S), 5 W avg / 12 W peak | Manual v1.10, p.16 |
| Camera USB-C | Firmware upgrade only - not power, not video | Manual + empty dmesg |
| Gimbal control | SIYI SDK over UDP, same Ethernet link | Manual v1.10 |

The 16 GB board settles the model choice: Qwen2.5-VL-3B at INT4 fits comfortably alongside the TensorRT engines and ROS 2. The 8 GB fallback ladder is not needed.

## Wiring conclusion

Every device taps the battery in parallel. Nothing daisy-chains through the Pixhawk.

LiPo main lead (XT60) -+-> power module --(6-pin JST-GH)--> Pixhawk POWER1  [5 V]

+-> ESCs / motors                                    [pack V]

+-> SIYI A8 mini power cable            [11-25.2 V, own branch]

+-> DC regulator ----------> Jetson Orin NX   [carrier-dependent]

LiPo balance lead (5-pin JST-XH) --> charger only. Never a power source.

Data: A8 mini --Ethernet--> Jetson      [RTSP video + gimbal UDP, one cable]

Pixhawk TELEM1 ------> Jetson      [MAVLink / MAVROS, read-only]

A8 mini UART --------> TELEM2      [optional: attitude feed + photo EXIF]

## Four mistakes identified and avoided

The 5-pin balance lead is not a power source. It is a per-cell tap for the charger. Thin wire, and which pins you use decides whether you get 3.7, 7.4, 11.1 or 14.8 V.

The Pixhawk POWER port is an input. It takes 5 V into the flight controller from the power module. There is no 12 V rail to draw from, and it cannot feed a 12 W gimbal.

USB-C will not power or stream the camera. Firmware upgrades only. Confirmed by an empty dmesg on the Jetson.

Do not run the camera through the PDB. SIYI is explicit: power the gimbal directly from a 3S-6S battery if the aircraft rolls hard.

# 3. Design decisions and rationale

## 3.1 Detector — YOLO11-pose

One pose model rather than a detector plus a pose model: the pose variant already emits person boxes, so running both would double the cost for nothing. ByteTrack for identity because it keeps low-confidence boxes alive, which is exactly the regime small aerial targets live in.

Rejected: YOLOv8 (superseded), RT-DETR (heavier at our input sizes), NVIDIA PeopleNet (ground-level viewpoint bias), any cloud API (violates the offline constraint).

Open licensing risk: Ultralytics YOLO is AGPL-3.0. Acceptable for an open academic project, contagious if RAPTOR ships to a third party without source. This must be decided before code is built on it.

## 3.2 Posture — a small model we build ourselves

Posture is a geometry question and gets a geometric answer: the torso vector from 17 COCO keypoints. Deterministic, explainable to an operator, effectively free, and it cannot hallucinate.

The non-obvious part is gravity alignment. During a banked turn the whole image rotates, so a standing person's torso reads as horizontal and a naive classifier raises a false casualty. We rotate keypoints into a gravity-aligned frame using the Pixhawk's own roll estimate, arriving over MAVROS, before classifying.

No public model does this, because no public model has our IMU stream. This is the genuinely novel and publishable component of the project.

## 3.3 Scene understanding — a VLM, not an LLM

A text-only language model cannot see the camera; the best it could do is paraphrase the detector's labels, which adds nothing. The value — dark staining on a trouser leg, an arm bent unnaturally, a person trapped under debris — lives in pixels no detector describes.

| Model | Role | Reasoning |
| --- | --- | --- |
| Qwen2.5-VL-3B | Primary, INT4 | Fits in ~2-2.5 GB. Best small model for fine visual detail, real region grounding, native dynamic resolution. Verify the checkpoint licence. |
| VILA-1.5-3B | Fallback 1 | NVIDIA-optimised for Jetson; easiest deployment path. |
| SmolVLM2 | Fallback 2 | Apache-2.0, tiny and fast; shallower descriptions. |
| GPT-4o / Claude / Gemini | Rejected | Best quality, but a disaster area has no reliable link. Optional ground-station enrichment only, never flight-critical. |
| LLaVA-1.6 7B | Rejected | Poor size-to-quality trade against a 2025-era 3B. |

## 3.4 Build, fine-tune, or use off the shelf

| Component | Verdict |
| --- | --- |
| Person detector | Fine-tune a COCO-pretrained backbone on aerial SAR data. Training from scratch would be strictly worse and cost a semester. |
| Pose estimator | Use off the shelf. |
| Posture classifier | Build it. Needs our IMU-derived gravity alignment; nothing public has that. |
| Action recognition | Defer. Posture plus motion may already cover the critical cases. |
| VLM | Use off the shelf. We lack the data to fine-tune, and a LoRA on a few hundred staged casualties would learn our volunteers' clothing, not injury. |

Fine-tuning the detector is not optional: stock YOLO trained on ground-level photographs systematically misses prone people seen from above, which is the exact failure we can least afford. Candidate datasets: HERIDAL, SARD, Okutama-Action, VisDrone, TinyPerson.

# 4. Key finding — the altitude ceiling is lower than assumed

The A8 mini's 81 degree horizontal FOV gives a focal length of about 1124 px across a 1920 px frame. Geometric prediction, nadir view, no zoom:

| Altitude | Standing (1.7 m) | Prone from overhead (0.45 m) | Ground swath |
| --- | --- | --- | --- |
| 20 m | 96 px | 25 px | 34 m |
| 25 m | 76 px | 20 px <- detection floor | 43 m |
| 30 m | 64 px | 17 px | 51 m |
| 40 m | 48 px | 13 px | 68 m |
| 60 m | 32 px | 8 px | 103 m |

A prone casualty falls below the ~20 px detection floor at roughly 25 m. The wide lens buys coverage and costs pixels. If this holds in measurement it is the most operationally important number the project will produce, and it forces a choice: fly lower than the coverage-optimal altitude, or adopt tiled inference.

Note: earlier drafts assumed a 60 degree FOV. Those figures were corrected today once the manual was consulted. The real optics make the problem harder, not easier.

# 5. Environment bring-up

Work begun on the Jetson the same day, ahead of the camera arriving on power.

## Resolved: externally-managed-environment (PEP 668)

JetPack 6 is Ubuntu 22.04, which blocks pip install into the system Python. Fixed with a virtual environment:

python3 -m venv --system-site-packages ~/raptor-venv

source ~/raptor-venv/bin/activate

The --system-site-packages flag is mandatory, not stylistic. JetPack's CUDA-enabled PyTorch, TensorRT and OpenCV live in the system site-packages and cannot be reinstalled from PyPI on aarch64, where the torch wheel is CPU-only. A sealed venv imports cleanly and silently never touches the GPU. Using --break-system-packages was rejected as a way to corrupt the JetPack install.

## Standing check after any install that pulls torch

python -c "import torch; print(torch.cuda.is_available())"

Must print True. Ultralytics lists torch as a dependency and pip can quietly substitute a CPU wheel.

## Reboot behaviour recorded

| Lost on reboot | Restore with |
| --- | --- |
| Venv activation | source ~/raptor-venv/bin/activate |
| jetson_clocks | sudo jetson_clocks - never persists, by design |
| Camera static IP | sudo ip addr add 192.168.144.30/24 dev eth0, or netplan |

nvpmodel does persist. Model weights are ordinary files and persist. Note that yolo is a one-shot command, not a background service — which is why the production system will be a long-running ROS 2 node that loads the model once.

# 6. Current blocker

pip install ultralytics reports numpy and scipy not installed.

Diagnose first — the venv may be sealed off from JetPack's packages:

grep system-site ~/raptor-venv/pyvenv.cfg

# must read: include-system-site-packages = true

source ~/raptor-venv/bin/activate

pip install "numpy<2" scipy

python -c "import torch, numpy; print(torch.__version__, numpy.__version__, torch.cuda.is_available())"

Pin numpy below 2.0. NVIDIA's Jetson PyTorch wheels are built against numpy 1.x, and numpy 2.0 changed the C ABI — installing numpy 2 is a common way to silently break CUDA torch on this board.

Second blocker: the camera has never been powered. Waiting on a 12 V bench supply (2 A current limit). No ping, no stream and no decode has been tested.

# 7. Deliverables produced today

| File | Contents |
| --- | --- |
| ProgressReports/2026-09-16-raptor-jetson-perception-design.md | Full design report |
| ProgressReports/STATUS.md | Living where-we-stopped checkpoint |
| RAPTOR/docs/01-system-overview.md | Mission, hardware, constraints, data flow |
| RAPTOR/docs/02-detection-and-pose.md | Detector and posture choices |
| RAPTOR/docs/03-scene-understanding-vlm.md | VLM selection, prompting, output schema |
| RAPTOR/docs/04-ros2-architecture.md | Node graph, topics, custom messages |
| RAPTOR/docs/05-custom-models-and-data.md | Build vs fine-tune vs use; datasets |
| RAPTOR/docs/06-benchmark-plan.md | Experiments E1-E8, performance budget |
| RAPTOR/docs/07-roadmap.md | Phases 0-6, milestones, risks |
| RAPTOR/docs/08-limitations-and-safety.md | Failure modes, privacy, GACA, model card |
| RAPTOR/docs/09-jetson-environment.md | Venv, PEP 668, CUDA torch, board commands |
| RAPTOR/docs/raptor-explainer.html | Animated explainer with interactive figures |
| RAPTOR/docs/raptor-wiring-map.html | Interactive harness diagram |

Six commits pushed to main at github.com/aniskoubaa/alfaisalx_students_projects (a9f346f..f3944c8). Approximately 4,000 lines.

# 8. Open questions

| # | Question | Blocks |
| --- | --- | --- |
| 1 | Which carrier board is the Orin NX on? | Regulator voltage: 5, 12 or 19 V |
| 2 | ArduPilot or PX4 on the Pixhawk 6C? | Optional gimbal UART parameters |
| 3 | Actual streamed resolution and frame rate | Altitude table; detector input size |
| 4 | H.264 or H.265 on the stream? | GStreamer pipeline elements |
| 5 | Is AGPL-3.0 acceptable for this project? | Whether we build on YOLO11 at all |
| 6 | Rename jetson_orin_nano_benchmarks/? | Flight computer is an NX, not a Nano |

# 9. Next steps

Immediate, in order:

Fix the numpy/scipy blocker; confirm torch.cuda.is_available() is True.

Run yolo checks; confirm it names the GPU rather than cpu.

Measure the FP16 PyTorch baseline: yolo benchmark model=yolo11n-pose.pt imgsz=640, at MAXN, with tegrastats sampling in a second terminal.

Acquire a 12 V bench supply and power the camera; confirm the self-test sweep and a solid green status LED.

Configure 192.168.144.30 on eth0; ping the camera; open the RTSP stream.

Prove hardware decode with a GStreamer pipeline using nvv4l2decoder.

Install ROS 2 Humble; bring up MAVROS on Pixhawk TELEM1.

Record a rosbag and replay it — the Phase 0 finish line.

Start data-collection planning in parallel. The consent form, flight plan across three altitudes and lighting conditions, and the labelling workflow are the long pole for Phases 3-5 and take longer than the code.

# 10. Scope and standing caveat

RAPTOR perceives and reports. It does not fly the aircraft, plan paths, or command the flight controller — the Jetson reads telemetry from MAVROS and never writes control commands. Enforced in code, so a perception bug can never become a flight safety incident.

The system is a search aid, not a medical device and not a triage system. The VLM produces observations; the triage flag is computed by explicit, auditable rules over structured fields, each carrying a plain-words reason. A human decides what happens. Placeholder thresholds need review by someone with real search-and-rescue or emergency-medicine experience before any operational use.

Every performance figure in this report is a target, not a measurement. Nothing has been benchmarked yet. The altitude table is geometry, not data. No figure here belongs in a presentation as a finding until it comes out of a recorded run.

RAPTOR / AlfaisalX — report generated 2026-09-16 22:54